The Bombay Salesian Society's Don Bosco Institute of Technology (An Autonomous Institute Affiliated to University of Mumbai)

Mid-Semester Examination

25ET4PCC02: Electromagnetics & Antennas Max Marks: 30 Date: _______________ Time: 10.00 – 11.30 am

- Assume appropriate data wherever necessary. - Attempt any ONE from Q1 A and Q1 B. - Attempt any ONE from Q2 A and Q2 B. - Attempt any ONE from Q3 A and Q3 B.

Course Outcomes

CO1: Define and state fundamental concepts of static & time-varying EM fields, Maxwell's equations, computational methods, and antenna parameters.

CO2: Explain the significance and interrelationships of EM field concepts, computational methodologies, and performance parameters of antennas.

CO3: Apply basic EM concepts and computational techniques to solve EM problems, analyze antenna performance, and visualize field distributions.

| Q. No. | Questions | Marks | CO |
| --- | --- | --- | --- |
| Q1 A (i) | State and explain the four Maxwell's equations in point (differential) form. State the physical significance of each equation. | 5 | CO2 |
| Q1 A (ii) | Explain Gauss's Law and Faraday's Law in both integral and differential form. Mention the shortcoming of each form. | 5 | CO2 |
|  | (or) |  |  |
| Q1 B (i) | State Poynting Theorem. Write the final expression and explain the physical meaning of each term. What is the significance of the Poynting vector? | 5 | CO2 |
| Q1 B (ii) | Explain the difference between conduction current and displacement current with the aid of Maxwell's equations. Why did Maxwell add the displacement current term? | 5 | CO2 |
| Q2 A | (i) Derive the Continuity Equation and explain its physical relevance. (5M) (ii) Explain Laplace's and Poisson's equations. Write expressions in Cartesian and Cylindrical coordinates and state their applications. (5M) | 10 | CO1 |
|  | (or) |  |  |
| Q2 B | (i) Derive an expression for Electric Field Intensity E due to an infinite line charge of density ρl using Gauss's Law. (5M) (ii) Justify that magnetic flux lines do not diverge. Which Maxwell's equation supports this? Explain using the concept of magnetic monopoles. (5M) | 10 | CO1 |
| Q3 A | Four like charges of 40 µC each are located at the four corners of a square whose diagonal is 12 m. Find the force on a 200 µC charge located 5 m above the centre of the square. | 10 | CO3 |
|  | (or) |  |  |
| Q3 B | In free space, V = 6xy²z + 8. At point P(1, 2, −5) find: (i) Electric Field Intensity E (5M) (ii) Volume Charge Density ρv (5M) | 10 | CO3 |

MODEL ANSWERS  —  E&A MSE

Q1 A(i)  Maxwell's Equations in Point Form  [5 Marks | CO2]

Maxwell's equations are the four fundamental laws governing classical electromagnetism. In point (differential) form:

1.  ∇ · D = ρv   (Gauss's Law for E-field)

D = electric flux density; ρv = volume charge density. Electric flux lines originate from +ve charges and terminate at −ve charges. Derived from Coulomb's law.

2.  ∇ · B = 0   (No Magnetic Monopoles)

B has zero divergence everywhere. Magnetic flux lines always form closed loops — no isolated magnetic poles exist.

3.  ∇ × E = −∂B/∂t   (Faraday's Law)

A time-varying B induces a circulating E at every point. Negative sign = Lenz's law. For static fields: ∇×E = 0 → E is conservative.

4.  ∇ × H = J + ∂D/∂t   (Modified Ampere's Law)

Conduction current J and displacement current ∂D/∂t both produce magnetic field. Maxwell added ∂D/∂t to keep consistency with the continuity equation. Predicts EM wave propagation at c = 1/√(μ₀ε₀) ≈ 3×10⁸ m/s.

Q1 A(ii)  Gauss's Law & Faraday's Law  [5 Marks | CO2]

Gauss's Law (Electric):

Integral form:    ∮ D · dS = Q_enc

Point form:       ∇ · D = ρv

Shortcoming (integral): Gives only net flux; no local field info; works only for symmetric geometries.

Shortcoming (differential): Requires field differentiability; not valid at discontinuities.

Faraday's Law:

Integral form:    EMF = ∮ E · dl = −d/dt ∫∫ B · dS

Point form:       ∇ × E = −∂B/∂t

Shortcoming (integral): Only net EMF around a loop; path geometry may be complex; no point-wise information.

Shortcoming (differential): Not valid at material boundaries; requires vector calculus; assumes macroscopic continuum.

Q1 B(i)  Poynting Theorem  [5 Marks | CO2]

Statement: The net EM power flowing INTO a closed volume = rate of increase of stored EM energy + ohmic power dissipated.

Poynting Vector:   P = E × H   [W/m²]

Represents instantaneous EM power per unit area. Direction of P = direction of wave propagation.

Final Expression:

−∮(E×H)·dS  =  ∂/∂t ∫∫∫(½εE² + ½μH²)dV  +  ∫∫∫ σE² dV

Meaning of each term:

•  −∮(E×H)·dS  :  Net EM power flowing INTO the closed volume.

•  ½εE²  :  Electric energy density stored [J/m³].

•  ½μH²  :  Magnetic energy density stored [J/m³].

•  ∂/∂t ∫∫∫(½εE²+½μH²)dV  :  Rate of increase of total stored EM energy.

•  ∫∫∫ σE² dV  :  Ohmic (Joule's) power dissipated as heat.

Significance: Used for antenna radiation power (∮P·dS), verifying energy conservation, and waveguide analysis. In lossless media, power only flows and stores → EM wave propagation.

Q1 B(ii)  Conduction vs Displacement Current  [5 Marks | CO2]

From Maxwell's 4th equation:  ∇ × H = J + ∂D/∂t  =  Jc + Jd

Conduction Current  Jc = σE:

•  Physical flow of free electrons/ions under E-field. Exists only in conductors (σ ≠ 0).

•  Obeys Ohm's law. Causes I²R (Joule) heating. In phase with E.

•  Example: Current in a copper wire.

Displacement Current  Jd = ε∂E/∂t:

•  NOT actual charge flow — a field quantity due to time-varying E-field.

•  Exists in dielectrics, free space, and vacuum. Does NOT cause Joule heating.

•  Leads Jc by 90°.  Example: 'Current' through a capacitor's dielectric gap.

Why Maxwell added Jd:

Without ∂D/∂t, Ampere's law gives ∇·J = 0 always — contradicting the continuity equation ∇·J = −∂ρv/∂t for time-varying situations. Adding Jd restores consistency and allows EM wave prediction in free space.

Q2 A  Continuity Equation + Laplace & Poisson's Equations  [10 Marks | CO1]

(i) Continuity Equation  [5 M]

Consider volume V enclosed by surface S. Total outward current:

∮ J · dS = −d/dt ∫∫∫ ρv dV    (outflow reduces enclosed charge)

Applying Divergence theorem:  ∫∫∫ (∇·J) dV = −∫∫∫ (∂ρv/∂t) dV

For arbitrary volume:

∇ · J + ∂ρv/∂t = 0      ← Continuity Equation

Alternative derivation via Maxwell:

∇×H = J + ∂D/∂t → take divergence → ∇·J + ∂(∇·D)/∂t = 0 → ∇·J + ∂ρv/∂t = 0  ✓

Physical Relevance:

•  Expresses conservation of charge — charge cannot be created or destroyed.

•  Steady state: ∂ρv/∂t = 0 → ∇·J = 0 (no divergence of steady current).

•  Validates Maxwell's addition of displacement current.

•  Governs charge relaxation: time constant τ = ε/σ.

(ii) Laplace's and Poisson's Equations  [5 M]

From ∇·D = ρv, D = εE, E = −∇V:

∇·(ε(−∇V)) = ρv  →  ∇²V = −ρv/ε   ← Poisson's Equation

When ρv = 0:     ∇²V = 0                ← Laplace's Equation

In Cartesian:    ∇²V = ∂²V/∂x² + ∂²V/∂y² + ∂²V/∂z²

In Cylindrical:  ∇²V = (1/r)∂/∂r(r∂V/∂r) + (1/r²)∂²V/∂φ² + ∂²V/∂z²

Applications:

•  Poisson's: Potential in regions with space charge (electron beams, semiconductors).

•  Laplace's: Potential between capacitor plates, coaxial structures.

•  Uniqueness theorem: solution satisfying boundary conditions is unique.

Q2 B  E due to Infinite Line Charge + No Divergence of B  [10 Marks | CO1]

(i) E due to Infinite Line Charge ρl  [5 M]

Consider infinite line charge with density ρl (C/m) along z-axis.

Gaussian Surface: Coaxial cylinder of radius r, height L.

Applying Gauss's law:   ∮ D · dS = Q_enc = ρl · L

Top & bottom caps: D ⊥ dS → zero contribution.

Curved surface: D ∥ dS (both radial):    D × (2πrL) = ρl × L

D = ρl / (2πr)  aᵣ   [C/m²]

E = ρl / (2πε₀r)  aᵣ   [V/m]

Key observations:

•  E ∝ 1/r (compare: point charge E ∝ 1/r²).

•  E is purely radial — no axial or angular component by symmetry.

•  In dielectric: E = ρl/(2πεr) where ε = ε₀εᵣ.

(ii) Magnetic Flux Lines Do Not Diverge  [5 M]

Maxwell's equation:  ∇ · B = 0

By Gauss's Divergence theorem:   ∮ B · dS = ∫∫∫ (∇·B) dV = 0

This means: the net magnetic flux through any closed surface is always zero → field lines that enter must also exit → flux lines form closed loops.

Physical Justification — No Magnetic Monopoles:

•  Electric field lines diverge from +ve charges because isolated electric charges (monopoles) exist.

•  No isolated magnetic charge (monopole) has ever been observed. Every magnet has both N and S poles. Even breaking a magnet produces two smaller complete magnets.

•  Hence, there are no sources/sinks of B — field lines always close on themselves. They do not diverge.

Consequence: B = ∇×A (vector potential), since ∇·(∇×A) = 0 identically — consistent with ∇·B = 0.

Q3 A  Four Charges at Square Corners  [10 Marks | CO3]

Given:

•  Four charges Q = 40 µC = 40×10⁻⁶ C at four corners of a square.

•  Square diagonal = 12 m  →  side = 12/√2 = 6√2 m ≈ 8.485 m.

•  Test charge Q₀ = 200 µC = 200×10⁻⁶ C at point P, 5 m above the centre.

Step 1 — Geometry:

Half-diagonal = 6 m = distance from each corner to the centre (in xy-plane).

Distance from each corner charge to point P:

r = √(6² + 5²) = √(36 + 25) = √61 ≈ 7.81 m

Step 2 — Force from each charge (Coulomb's Law):

F = kQQ₀/r²  =  (9×10⁹ × 40×10⁻⁶ × 200×10⁻⁶) / 61

F  =  72/61  ≈  1.180 N  (per corner charge)

Step 3 — Resolve into components:

By symmetry, all horizontal (xy-plane) components from the 4 charges cancel.

Only the vertical (z) components add:

cos θ = h/r = 5/√61 = 0.6402

F_z per charge = 1.180 × 0.6402 = 0.755 N

Step 4 — Total Force:

F_total = 4 × 0.755 ≈ 3.02 N  (+z direction, vertically upward)

∴  Force on the 200 µC charge = 3.02 N  directed vertically upward (away from the square plane).

Q3 B  V = 6xy²z + 8: Find E and ρv at P(1, 2, −5)  [10 Marks | CO3]

Given:  V = 6xy²z + 8  in free space,  Point P(1, 2, −5)

Part (i) — Electric Field  E = −∇V  [5 M]

Partial derivatives:

∂V/∂x = 6y²z,       at P: 6(4)(−5) = −120

∂V/∂y = 12xyz,       at P: 12(1)(2)(−5) = −120

∂V/∂z = 6xy²,        at P: 6(1)(4) = +24

E = −∇V = −[(−120)ax + (−120)ay + (24)az]

E = 120 ax + 120 ay − 24 az   V/m     |E| = √(14400+14400+576) ≈ 171.4 V/m

Part (ii) — Volume Charge Density  ρv  [5 M]

From Poisson's equation:   ∇²V = −ρv/ε₀

∂²V/∂x² = 0,   ∂²V/∂y² = 12xz,   ∂²V/∂z² = 0

∇²V = 12xz

At P(1, 2, −5):   ∇²V = 12(1)(−5) = −60

ρv = −ε₀ × ∇²V = −(8.854×10⁻¹²)(−60)

ρv = 531.24 pC/m³  ≈  0.531 nC/m³